CENTRO DE ENSEÑANZA TÉCNICA Y SUPERIOR

LCD & Push-Button Integration in the STM32

Automated Irrigation System

Teacher: Carlos Alberto Villareal Molina

Oskar Liborio García Velíz — 38265

Carlos Humberto Araiza Quintana — 36986

Luis Angel Lugo Muñiz — 37915

Sebastian Lizalde Herrera — 38241

Tijuana, B.C. — May 16th, 2026

LCD & Push-Button Integration in the STM32

This document describes how the 16x2 LCD display with I2C backpack module and the three push-buttons are integrated into the STM32F411RE microcontroller for the automated irrigation system. The LCD display provides real-time feedback to the user, showing sensor readings, system status, and the active irrigation section. The push-buttons allow the user to manually select which irrigation section to activate and to toggle between automatic and manual operating modes.

- LCD Display with I2C Module

  - Operating Principle

The display used is a standard 16x2 LCD with an HD44780-compatible controller. In its native parallel interface, the LCD requires at least 6 GPIO pins from the microcontroller to operate. To preserve GPIO pins for the sensors and actuators, a PCF8574 I2C backpack module is soldered onto the LCD, reducing the interface to only two signal lines, SDA and SCL, shared on the I2C bus.

The PCF8574 is an 8-bit I/O expander that receives I2C commands from the STM32 and translates them into the parallel signals required by the HD44780 LCD controller. Communication between the STM32 and the PCF8574 follows the standard I2C protocol: the STM32 acts as the master, the PCF8574 acts as the slave, and each byte sent over SDA controls the 8 output pins of the expander, which in turn drive the LCD's data and control lines.

The default I2C address of the PCF8574 is 0x27 or 0x3F, depending on the manufacturer. The address can be identified by running an I2C scanner on the STM32, which cycles through all 128 possible addresses and checks which ones produce an acknowledgment on the bus.

  - STM32 Peripheral: I2C1

The LCD is connected to the STM32 through the I2C1 peripheral, using pins PB8 (I2C1_SCL) and PB9 (I2C1_SDA). These pins must be configured in Alternate Function mode with the open-drain output type, which is required by the I2C standard. According to the STM32 Nucleo-64 user manual (UM1724, Table 10), using PB8 and PB9 as I2C1 requires enabling solder bridges SB46 and SB52 on the board while disabling SB51 and SB56.

  - Bare Metal I2C1 Configuration

The following steps describe the complete register-level configuration of I2C1 for communication with the PCF8574 at standard speed (100 kHz):

Step 1 — Enable clocks: Enable the GPIOB clock and the I2C1 clock through the RCC registers.

RCC->AHB1ENR |= RCC_AHB1ENR_GPIOBEN;  // GPIOB clock

RCC->APB1ENR |= RCC_APB1ENR_I2C1EN;   // I2C1 clock

Step 2 — Configure PB8 and PB9 as Alternate Function, Open-Drain:

// PB8 and PB9 → Alternate function mode (MODER bits = 10)

GPIOB->MODER &= ~((3U << 16) | (3U << 18));

GPIOB->MODER |=  ((2U << 16) | (2U << 18));

// Open-drain output type (required by I2C)

GPIOB->OTYPER |= (1U << 8) | (1U << 9);

// High-speed GPIO output (needed at 100 kHz)

GPIOB->OSPEEDR |= (3U << 16) | (3U << 18);

// Pull-up on both SDA and SCL

GPIOB->PUPDR &= ~((3U << 16) | (3U << 18));

GPIOB->PUPDR |=  ((1U << 16) | (1U << 18));

// AF4 = I2C1 on PB8 and PB9 (AFRH bits)

GPIOB->AFR[1] &= ~((0xFU << 0) | (0xFU << 4));

GPIOB->AFR[1] |=  ((4U  << 0) | (4U  << 4));

Step 3 — Configure I2C1 registers: Disable I2C1 before configuration, set the peripheral clock frequency, configure the clock control register for 100 kHz standard mode, and set the rise time register.

I2C1->CR1   &= ~I2C_CR1_PE;        // Disable I2C1 during configuration

I2C1->CR2    = 16;                  // APB1 clock = 16 MHz

I2C1->CCR    = 80;                  // 100 kHz: CCR = 16MHz / (2 × 100kHz)

I2C1->TRISE  = 17;                  // TRISE = (1000ns / 62.5ns) + 1 = 17

I2C1->CR1   |= I2C_CR1_PE;         // Enable I2C1

  - Sending Data to the LCD via I2C

Writing to the LCD through the PCF8574 requires following the HD44780 4-bit initialization sequence and then sending each character or command as two nibbles. Each byte sent to the PCF8574 over I2C directly controls its 8 output pins, which map to the LCD's RS, RW, EN, and D4–D7 lines plus the backlight control bit.

A complete I2C write transaction to the PCF8574 consists of generating a START condition, sending the slave address with the write bit, sending the data byte, and generating a STOP condition:

Generate START condition:

I2C1->CR1 |= I2C_CR1_START;

while (!(I2C1->SR1 & I2C_SR1_SB));       // Wait for START generated

Send slave address (write):

I2C1->DR = (PCF8574_ADDR << 1) | 0;      // Address + write bit

while (!(I2C1->SR1 & I2C_SR1_ADDR));     // Wait for address ACK

(void)I2C1->SR2;                          // Clear ADDR flag

Send data byte:

I2C1->DR = data;                          // PCF8574 output byte

while (!(I2C1->SR1 & I2C_SR1_TXE));      // Wait for transmit buffer empty

Generate STOP condition:

I2C1->CR1 |= I2C_CR1_STOP;

To send a character or command to the HD44780, this I2C write function is called twice per byte once for the upper nibble and once for the lower nibble, with the EN (enable) bit toggled between each write to latch the data into the LCD controller. The RS bit in the PCF8574 output byte selects whether the byte is interpreted as a command (RS = 0) or a display character (RS = 1).

The LCD is initialized with the standard HD44780 4-bit initialization sequence: function set (4-bit mode, 2 lines, 5x8 font), display on with cursor off, entry mode set (increment, no shift), and clear display. This sequence must be executed with precise timing delays between commands, implemented using TIM2 as a microsecond counter.

  - Display Content

The LCD shows real-time system information updated on every main loop iteration. The display is divided into two lines as follows:

- Line 1 — Shows the soil moisture percentage of the currently active section and the ambient temperature read from the DHT11 sensor.

- Line 2 — Shows the water level in the tank (from the JSN-SR04T) and the current system mode (AUTO or MANUAL) along with which section is active (A or B).

To update the display, the cursor is repositioned using the Set DDRAM Address command before writing each value, avoiding a full display clear which causes visible flickering. Only the numeric values are rewritten on each update cycle, while static labels such as 'Hum:', 'Temp:', 'Tank:', and 'Mode:' are written once during initialization.

  - Key Specifications

| Parameter | Value |
| --- | --- |
| Display | 16 columns × 2 rows (16x2 LCD) |
| Controller | HD44780-compatible |
| I2C adapter | PCF8574 I/O expander backpack |
| I2C address | 0x27 or 0x3F (depending on PCF8574 variant) |
| STM32 pins | PB8 (I2C1_SCL), PB9 (I2C1_SDA) |
| Power supply | VCC: 5V, Logic: 3.3V compatible via PCF8574 |
| Contrast | Adjustable via on-board potentiometer |
| Pin configuration | GND, VCC (5V), SDA, SCL |

  - Important Considerations

- The LCD VCC must be connected to 5V, not 3.3V. The PCF8574 backpack and the LCD backlight require 5V to operate correctly. The SDA and SCL lines, however, operate at 3.3V because the STM32's GPIO output voltage is 3.3V and the PCF8574 is tolerant of this level.

- External pull-up resistors (typically 4.7 kΩ) on SDA and SCL are required for reliable I2C operation. The internal pull-ups enabled through the STM32 PUPDR register may be sufficient at 100 kHz and short cable lengths, but external resistors are recommended for robustness.

- If the I2C address is unknown, run an I2C address scanner at startup: iterate from address 0x08 to 0x77, send a START + address + STOP for each, and record which addresses return an ACK. The PCF8574 will be either 0x27 or 0x3F.

- LCD contrast is adjusted via the on-board potentiometer on the PCF8574 module. If the display shows no characters after initialization, the contrast setting is the first thing to check.

- Avoid calling the LCD update function too frequently. The HD44780 requires a minimum execution time per command (37 µs for most commands, 1.52 ms for clear display). Updating the display on every main loop iteration is only safe if the loop period is long enough; otherwise, use a TIM2-based timer flag to limit updates to every 500 ms.

- Push-Buttons

  - Role in the System

Three push-buttons are used in the automated irrigation system. Button 1 (PA0) manually selects Section A for irrigation, Button 2 (PA1) manually selects Section B, and Button 3 (PA4) toggles the system between automatic mode, where irrigation decisions are made based on soil moisture readings, and manual mode where the user directly controls which section is irrigated by pressing the corresponding section button.

In automatic mode, the push-buttons for section selection are ignored; the system reads the YL-69 sensors and irrigates whichever section has moisture below the defined threshold. In manual mode, the system disregards sensor readings and activates the valve and pump for whichever section the user selects by pressing the corresponding button.

  - Electrical Configuration

Each push-button is connected between the GPIO pin and GND. The STM32 internal pull-up resistor is enabled on each pin, which holds the pin at a logic HIGH (3.3V) when the button is not pressed. When the button is pressed, it shorts the pin to GND, driving it to logic LOW. The firmware detects a button press on the falling edge of the signal, the transition from HIGH to LOW.

  - Bare Metal GPIO and EXTI Configuration

The three button pins are configured as GPIO inputs with internal pull-up resistors enabled. External interrupts (EXTI) are configured on each pin so that the MCU responds immediately to a button press without polling the pin state in the main loop.

Step 1 — Enable GPIOA clock:

RCC->AHB1ENR |= RCC_AHB1ENR_GPIOAEN;

Step 2 — Configure PA0, PA1, PA4 as inputs with pull-up:

// MODER: 00 = input (default, but set explicitly for clarity)

GPIOA->MODER &= ~((3U << 0) | (3U << 2) | (3U << 8));

// Enable internal pull-up on PA0, PA1, PA4

GPIOA->PUPDR &= ~((3U << 0) | (3U << 2) | (3U << 8));

GPIOA->PUPDR |=  ((1U << 0) | (1U << 2) | (1U << 8));

Step 3 — Configure SYSCFG to map EXTI lines to GPIOA: The SYSCFG peripheral routes the EXTI interrupt lines to a specific GPIO port. All three buttons are on GPIOA, so EXTI0, EXTI1, and EXTI4 must be mapped to Port A.

RCC->APB2ENR |= RCC_APB2ENR_SYSCFGEN;    // Enable SYSCFG clock

SYSCFG->EXTICR[0] &= ~(0xF << 0);        // EXTI0 → PA0 (bits [3:0] = 0000)

SYSCFG->EXTICR[0] &= ~(0xF << 4);        // EXTI1 → PA1 (bits [7:4] = 0000)

SYSCFG->EXTICR[1] &= ~(0xF << 0);        // EXTI4 → PA4 (bits [3:0] = 0000)

Step 4 — Configure EXTI for falling-edge trigger and unmask lines:

EXTI->FTSR |= (1U << 0) | (1U << 1) | (1U << 4);  // Falling edge trigger

EXTI->RTSR &= ~((1U << 0) | (1U << 1) | (1U << 4)); // Disable rising edge

EXTI->IMR  |= (1U << 0) | (1U << 1) | (1U << 4);  // Unmask EXTI lines

Step 5 — Enable EXTI interrupts in the NVIC:

NVIC_EnableIRQ(EXTI0_IRQn);    // PA0 — Button 1 (Section A)

NVIC_EnableIRQ(EXTI1_IRQn);    // PA1 — Button 2 (Section B)

NVIC_EnableIRQ(EXTI4_IRQn);    // PA4 — Button 3 (Mode toggle)

NVIC_SetPriority(EXTI0_IRQn, 2);

NVIC_SetPriority(EXTI1_IRQn, 2);

NVIC_SetPriority(EXTI4_IRQn, 2);

  - Interrupt Service Routines

Each button has its own Interrupt Service Routine (ISR). The ISR clears the EXTI pending flag, applies a software debounce check using TIM2, and then updates a global state variable that the main loop reads to determine the system's operating mode and active section.

Button 1 ISR — Section A selection (PA0):

void EXTI0_IRQHandler(void) {

if (EXTI->PR & (1U << 0)) {

EXTI->PR = (1U << 0);              // Clear pending flag

if (debounce_check()) {             // 50 ms debounce via TIM2

if (system_mode == MANUAL) {

active_section = SECTION_A;

}

}

}

}

Button 2 ISR — Section B selection (PA1):

void EXTI1_IRQHandler(void) {

if (EXTI->PR & (1U << 1)) {

EXTI->PR = (1U << 1);

if (debounce_check()) {

if (system_mode == MANUAL) {

active_section = SECTION_B;

}

}

}

}

Button 3 ISR — Mode toggle (PA4):

void EXTI4_IRQHandler(void) {

if (EXTI->PR & (1U << 4)) {

EXTI->PR = (1U << 4);

if (debounce_check()) {

system_mode = (system_mode == AUTO) ? MANUAL : AUTO;

}

}

}

  - Software Debounce

Mechanical push-buttons produce a noisy signal for a few milliseconds after each press and release due to the physical bouncing of the contacts. Without debouncing, a single press can generate multiple EXTI interrupts, causing unintended repeated mode toggles or section changes. A software debounce is implemented using TIM2 as a free-running microsecond counter.

The debounce function reads the current TIM2 count at the moment the ISR fires and compares it to the timestamp of the previous press on the same button. If less than 50 ms has elapsed since the last accepted press, the current event is discarded as a bounce. If 50 ms or more have elapsed, the press is accepted and the timestamp is updated.

uint32_t last_press_time = 0;

uint8_t debounce_check(void) {

uint32_t now = TIM2->CNT;              // TIM2 running at 1 MHz (1 tick = 1 µs)

if ((now - last_press_time) >= 50000) { // 50 ms = 50,000 µs

last_press_time = now;

return 1;                           // Valid press

}

return 0;                               // Bounce — ignore

}

  - Key Specifications

| Parameter | Value |
| --- | --- |
| Quantity | 3 push-buttons |
| STM32 pins | PA0 (Section A), PA1 (Section B), PA4 (Mode) |
| Configuration | GPIO Input with internal Pull-Up |
| Active state | LOW (button pressed pulls pin to GND) |
| Interrupt lines | EXTI0 (PA0), EXTI1 (PA1), EXTI4 (PA4) |
| Trigger edge | Falling edge (HIGH → LOW on press) |
| Debounce | Software — TIM2 counter-based delay (50 ms) |

  - Important Considerations

- Always clear the EXTI pending flag (EXTI->PR) at the beginning of each ISR before executing any logic. Failing to clear it will cause the ISR to re-enter immediately after returning, creating an infinite interrupt loop.

- Keep the ISR body as short as possible. Only update flags and state variables inside the ISR; all LCD updates, actuator control, and sensor reads should happen in the main loop reading those flags.

- The mode toggle button (PA4) must implement hysteresis in the main loop to prevent the system from rapidly cycling between AUTO and MANUAL if the button is held down. Only process the mode change flag once per press event.

- In automatic mode, pressing the section selection buttons (PA0 or PA1) must have no effect on the actuators. The ISR can still detect the press, but the mode check inside the ISR ensures the active_section variable is only updated when the system is in MANUAL mode.

- TIM2 must be initialized and running before the EXTI interrupts are enabled, since the debounce function reads TIM2->CNT. If TIM2 is not running at the time of the first button press, the debounce check will malfunction.

References

- STM32F411xC/xE Reference Manual (RM0383) — STMicroelectronics. https://www.st.com/resource/en/reference_manual/rm0383-stm32f411xce-advanced-armbased-32bit-mcus-stmicroelectronics.pdf

- STM32 Nucleo-64 User Manual (UM1724) — STMicroelectronics. https://www.st.com/resource/en/user_manual/um1724-stm32-nucleo64-boards-mb1136-stmicroelectronics.pdf

- STM32 I2C LCD Library Example — DeepBlueMbedded. https://deepbluembedded.com/stm32-i2c-lcd-library-example-16x2-20x4-multiple-lcds/

- HD44780 LCD Controller Datasheet — Hitachi.

- PCF8574 Remote 8-Bit I/O Expander Datasheet — Texas Instruments. https://www.ti.com/product/PCF8574