CENTRO DE ENSEÑANZA TÉCNICA Y SUPERIOR

Mechatronics Engineering | Microcontrollers and Embedded Systems

Peripherals and Pins as I/O

Automated Irrigation System

Professor: Carlos Alberto Villareal Molina

Oskar Liborio García Velíz — 38265

Carlos Humberto Araiza Quintana — 36986

Luis Angel Lugo Muñiz — 37915

Sebastian Lizalde Herrera — 38241

Tijuana, B.C. — April 25, 2026

- General Description

This report documents the pin and peripheral assignment of the STM32F411RET6 microcontroller (NUCLEO-F411RE board) for the automated irrigation system. The system operates two independent irrigation sections with continuous sensor monitoring and controlled actuation via servo motors and a relay module.

The pin assignment is based on the official NUCLEO-F411RE pinout (UM1724, Table 16 and Table 29), ensuring compatibility with the Arduino and morpho connectors available on the board.

- Bill of Materials (BOM)

The components that directly interact with the MCU are:

- STM32 Nucleo-64 (STM32F411RET6) — 1 unit

- YF-S201 flow sensor — 2 units

- JSN-SR04T ultrasonic sensor (tank water level) — 1 unit

- DHT11 temperature and humidity sensor — 2 units

- YL-69 soil moisture sensor — 2 units

- LCD display with I2C adapter (PCF8574) — 1 unit

- 5V Relay Module — 1 unit

- Solenoid valve (valves Section A and Section B) — 2 units

- Water pump — 1 unit

- Indicator LED — 2 units

- Push-buttons — 3 units

- Pin Assignment

Digital Inputs

Includes the three push-buttons for section selection and mode switching, the two YF-S201 flow sensors whose output is a pulse train captured by Input Capture on dedicated timers, and the ECHO pin of the JSN-SR04T ultrasonic sensor for monitoring the water level in the tank.

| Pin MCU | Signal | Peripheral | Mode | Component / Function |
| --- | --- | --- | --- | --- |
| PA0 | BTN_SECCION_A | GPIO / EXTI0 | Input Pull-Up | Push-button #1 — Section A selection |
| PA1 | BTN_SECCION_B | GPIO / EXTI1 | Input Pull-Up | Push-button #2 — Section B selection |
| PA4 | BTN_MODE | GPIO / EXTI4 | Input Pull-Up | Push-button #3 — Auto/manual mode switch |
| PA10 | US_ECHO | GPIO / TIM1_CH3 | Input Capture | JSN-SR04T — ECHO pin (tank water level) |
| PB3 | FLOW_A | GPIO / TIM2_CH2 | Input Capture | YF-S201 #1 — Flow sensor pulses in section A |
| PB4 | FLOW_B | GPIO / TIM3_CH1 | Input Capture | YF-S201 #2 — Flow sensor pulses in section B |

Analog Inputs — Soil Moisture Sensors (YL-69)

The YL-69 sensors deliver an analog voltage proportional to soil moisture. They are read via the STM32F411RE ADC1 on channels CH10 and CH11, accessible from the board's morpho connector CN7 (pins PC0 and PC1).

| Pin MCU | Signal | ADC Channel | Peripheral | Component / Function |
| --- | --- | --- | --- | --- |
| PC0 | SOIL_A | ADC1_CH10 | ADC1 | YL-69 #1 — Soil moisture Section A (0–3.3 V) |
| PC1 | SOIL_B | ADC1_CH11 | ADC1 | YL-69 #2 — Soil moisture Section B (0–3.3 V) |

Digital I/O — DHT11 Sensors

The DHT11 1-Wire protocol requires a GPIO pin configured as push-pull output for the start pulse and as input for data reception. Both sensors connect to port C pins accessible from the morpho connector CN7.

| Pin MCU | Signal | Function | Peripheral | Component / Function |
| --- | --- | --- | --- | --- |
| PC2 | DHT11_A | GPIO 1-Wire | GPIO Bidir. | DHT11 #1 — Temperature and humidity, Section A |
| PC3 | DHT11_B | GPIO 1-Wire | GPIO Bidir. | DHT11 #2 — Temperature and humidity, Section B |

Digital Outputs and PWM

The solenoid valves controlling each section receive a 50 Hz PWM signal with a duty cycle of 1 to 2 ms (0° to 180°). The water pump is activated through the relay module via a GPIO pin. The LEDs visually indicate which section is being irrigated. The JSN-SR04T TRIGGER pin emits a 10 µs pulse to start each level measurement.

| Pin MCU | Signal | Function AF | Peripheral | Component / Function |
| --- | --- | --- | --- | --- |
| PB6 | SERVO_A | TIM4_CH1 / AF2 | TIM4 | Solenoid valve #1 — Section A valve (PWM 50 Hz) |
| PC7 | SERVO_B | TIM3_CH2 / AF2 | TIM3 | Solenoid valve #2 — Section B valve (PWM 50 Hz) |
| PA8 | RELAY_BOMBA | GPIO | GPIO | Relay Module — Water pump activation |
| PA5 | LED_A | GPIO | GPIO | LED #1 — Active irrigation indicator Section A |
| PA6 | LED_B | GPIO | GPIO | LED #2 — Active irrigation indicator Section B |
| PA9 | US_TRIGGER | GPIO | GPIO | JSN-SR04T — Pin TRIGGER |

Communication — I2C

The LCD display communicates via I2C using the PCF8574 adapter, accessible on pins PB8 and PB9.

| Pin MCU | Signal | Function AF | Peripheral | Component / Function |
| --- | --- | --- | --- | --- |
| PB8 | I2C1_SCL | AF4 – I2C1 | I2C1 | LCD display with I2C module (PCF8574) — CLK |
| PB9 | I2C1_SDA | AF4 – I2C1 | I2C1 | LCD display with I2C module (PCF8574) — Data |

- Pin Assignment Summary

The following table consolidates all I/O categories with the total count of STM32F411RE pins and peripherals used in the project. Of the ~50 pins available on the board, the system uses 20, leaving ample room for future expansions.

| Category | Pins | Cant. | Peripherals used |
| --- | --- | --- | --- |
| Digital inputs (push-buttons, YF-S201 flow, ultrasonic echo) | PA0, PA1, PA4, PA10, PB3, PB4 | 6 | GPIO, EXTI, TIM2, TIM3 |
| Analog inputs (YL-69 soil moisture) | PC0, PC1 | 2 | ADC1 (CH10, CH11) |
| 1-Wire digital I/O (DHT11 temperature and humidity) | PC2, PC3 | 2 | Bidirectional GPIO |
| PWM outputs — Solenoid valves | PB6, PC7 | 2 | TIM3_CH2, TIM4_CH1 |
| Digital outputs — Relay (pump), LEDs and Trigger | PA8, PA5, PA6, PA9 | 4 | GPIO Output |
| Communication — LCD | PB8, PB9 | 4 | I2C |
| TOTAL PINS USED | 20 pins | 20 | GPIO, ADC1, TIM2/3/4, I2C |

- Implementation Notes

- The YF-S201 sensors generate pulses of up to 5 V; a voltage divider (e.g. 1 kΩ + 2 kΩ) is recommended before connecting to the STM32, whose maximum logic level is 3.3 V.

- The JSN-SR04T sensor also operates at 5 V. The ECHO pin must be protected with a voltage divider before connecting to pin PA10 of the microcontroller.

- The 5V relay module may require an active-low control signal. Verify the control polarity before powering the pump.

- PB8 and PB9 (I2C) are enabled via solder bridges SB46 and SB52 on the NUCLEO board (disabling SB51 and SB56), per Table 10 of UM1724.

- It is recommended to implement the firmware with interrupts for the flow sensors and ultrasonic echo.