CENTRO DE ENSEÑANZA TÉCNICA Y SUPERIOR

Designing of the Code that Makes the Hardware Work with the STM32

Automated Irrigation System

Teacher: Carlos Alberto Villareal Molina

Oskar Liborio García Velíz — 38265

Carlos Humberto Araiza Quintana — 36986

Luis Angel Lugo Muñiz — 37915

Sebastian Lizalde Herrera — 38241

Tijuana, B.C. — May 16th, 2026

Designing of the Code that Makes the Hardware Work with the STM32

This document describes the firmware developed for the automated irrigation system prototype, based on the STM32F411RE microcontroller (Nucleo-64 board). The code is organized in two functional groups: the first validates the flow sensor simulation and LCD display using PWM and I2C, and the second implements the water level measurement with the JSN-SR04T ultrasonic sensor and the water pump control. The firmware follows a layered driver architecture where low-level peripheral drivers (GPIO, TIM, I2C, UART, ADC) expose a clean API consumed by higher-level modules (LCD, flow sensor, ultrasonic, pump, button). The main loop coordinates all modules based on sensor readings and user input.

- Firmware Architecture and Module Overview

  - Flow Sensor Simulation and LCD

This project tests the valve-and-flow sensing chain and the LCD display together. A push-button on PC13 acts as a valve toggle: pressing it enables a PWM signal on PA6 (TIM3_CH1, 500 Hz, 70% duty) that simulates the valve opening and enables flow counting. The YF-S201 sensor signal is polled on PA0. Every second, the pulse count is converted to liters per minute and displayed both on the serial terminal (USART2) and on the 16×2 LCD via I2C1.

| Module / File | Responsibility |
| --- | --- |
| gpio_driver.c / .h | GPIO abstraction: port init, pin mode, set/clear/toggle/read, alternate function, pull-up/down |
| TIM_Driver.c / .h | Timer abstraction: init, set frequency or ms period, enable/disable, PWM compare modes and channels |
| pwm.c / .h | High-level PWM wrapper: configures TIM3_CH1 on PA6 (AF2), sets duty cycle as a percentage |
| flowsensor.c / .h | Flow sensor driver: polls PA0 on rising edge, accumulates pulse count, converts to L/min |
| i2c.c / .h | Bare metal I2C1 driver: init (PB8 SCL, PB9 SDA, 100 kHz), start/stop, address, write byte |
| lcd.c / .h | HD44780 LCD driver via PCF8574 I2C backpack: 4-bit mode, init, setCursor, writeString, writeNumber |
| systick.c / .h | SysTick 1 ms counter: used for non-blocking delays and polling intervals |
| uart.c / .h | USART2 TX driver: 115200 baud on PA2 (AF7), writeString, writeNumber |
| adc_driver.c / .h | ADC1 driver: single and injected conversions, configurable channel and sample time |
| utils.c / .h | Utility functions: snprintf, itoa, atoi, memCpy, memReverse |
| main.c | Application entry: initializes all peripherals, main loop handles button, PWM valve, flow count, LCD and UART updates |

  - Ultrasonic Water Level and Pump Control

This project implements the water level monitoring system and the automated pump control. The JSN-SR04T sensor is driven from PA9 (TRIG) and PB3 (ECHO) with TIM5 running as a free-running 1 MHz counter for microsecond timing. Distance is calculated from the ECHO pulse width, filtered through a 5-sample median and a spike rejection algorithm, and mapped to three water level states. The pump on PA5 is driven by a PWM signal whose duty cycle is adjusted automatically to 0%, 75%, or 100% according to the measured level. A single push-button on PA0 switches between automatic and manual modes, with click-and-hold detection. The LCD alternates two information pages every 2 seconds.

| Module / File | Responsibility |
| --- | --- |
| gpio_driver.c / .h | Refactored GPIO driver with GPIO_PinCfg_t struct (mode, pull, otype, speed) for full pin control |
| ultrasonic.c / .h | JSN-SR04T driver: TIM5 @1MHz, 5-sample median, spike filter, level classification (HIGH/MID/LOW/UNKNOWN) |
| pump.c / .h | Pump PWM driver: wraps pwm module, maps WaterLevel_t to 0/75/100% duty on TIM2_CH1/PA5 |
| button.c / .h | Push-button driver: EXTI0 on PA0, debounce via TIM5, click/hold detection, lcd_dirty flag, I2C protection |
| i2c_driver.c / .h | Upgraded I2C1 driver with bus recovery (i2c_recover), error flag handling, read/write with register address |
| lcd.c / .h | LCD with two alternating pages (status / data), mode transition messages, TIM5-based page timing |
| timer.c / .h | TIM4-based blocking delay module using tim_driver |
| uart.c / .h | USART2 driver (reused from Project 1) |
| main.c | Coordinates all modules: AUTO/MANUAL mode loop, sensor→pump→LCD→UART data flow, I2C bus protection |

- GPIO Driver

  - Overview

Both projects share the same GPIO abstraction concept but with slightly different APIs. Project 1 uses individual parameters for mode and pull-up/down, while Project 2 introduces a GPIO_PinCfg_t configuration struct that groups mode, output type, speed, and pull resistor into a single argument, making pin configuration more explicit and scalable.

In both versions, gpio_initPort() enables the corresponding RCC AHB1ENR clock bit for the requested port, and all pin operations use atomic BSRR writes for set and clear — avoiding read-modify-write race conditions on output pins.

  - Key Functions

gpio_initPort(port): Enables the peripheral clock for the specified GPIO port by setting the corresponding bit in RCC->AHB1ENR. Called before configuring any pin on that port.

gpio_setPinMode(port, pin, cfg): Configures the MODER, OTYPER, OSPEEDR, and PUPDR registers for the specified pin in a single function call using the GPIO_PinCfg_t struct (Project 2) or individual mode and pull parameters (Project 1).

gpio_setPin / gpio_clearPin(port, pin): Use the atomic BSRR register to drive a pin HIGH or LOW without reading ODR first, which prevents data hazards in interrupt-driven code.

gpio_readPin(port, pin, state): Reads the IDR register and writes the result to the caller-provided state pointer.

gpio_setAlternateFunction(port, pin, af): Sets MODER to alternate function mode (0x02) and writes the AF number into AFR[0] (pins 0–7) or AFR[1] (pins 8–15).

- Timer Driver and Time Management

  - TIM Driver

The TIM driver supports TIM1 through TIM5 and TIM9 through TIM11. tim_initTimer() enables the corresponding APB1 or APB2 clock and resets CR1, PSC, ARR, CNT, and SR to a known state. Two configuration modes are available:

- tim_setTimerMs(tim, ms): sets PSC to generate a 1 kHz tick (1 ms per count) and ARR to the desired period in milliseconds. Used for blocking delays and periodic event checking.

- tim_setTimerFreq(tim, freq): calculates PSC and ARR automatically to achieve the target frequency, iterating the prescaler until ARR fits within 16 bits. Used for PWM frequency configuration.

For PWM output, tim_setTimerCompareMode() configures the CCMRx register with PWM Mode 1 (output HIGH while CNT < CCR) and enables the output preload. tim_enableTimerCompareChannel() sets the CCxE bit in CCER to enable the channel output. The ARR preload is enabled through CR1_ARPE for glitch-free duty cycle updates.

  - SysTick (systick.c)

Project 1 uses the ARM SysTick peripheral as a 1 ms system tick counter. systick_init() calls SystemCoreClockUpdate() to get the actual MCU frequency and then SysTick_Config() to configure the reload value. The SysTick_Handler() ISR increments a volatile 32-bit counter. systick_getTick() returns the current count, which is used for non-blocking delays throughout the project:

uint32_t start = systick_getTick();

while ((systick_getTick() - start) < delay_ms) { }

  - TIM5 Free-Running Counter

Project 2 replaces SysTick with TIM5 configured as a 32-bit free-running microsecond counter (PSC = 15, giving 1 MHz tick from 16 MHz HSI). This allows microsecond-precision timing needed for the ultrasonic pulse measurement and the button debounce algorithm. TIM5 is initialized in ultrasonic_init() and shared by all modules that need µs timing:

TIM5->PSC = 15U;          // 16 MHz / 16 = 1 MHz → 1 tick = 1 µs

TIM5->ARR = 0xFFFFFFFFUL; // 32-bit free-running, wraps after ~71 min

TIM5->CR1|= TIM_CR1_CEN;

- PWM Module and Flow Sensor

  - PWM Driver

The PWM module uses TIM3 Channel 1 on PA6 (Alternate Function 2) to generate the valve control signal. pwm_init() configures the GPIO pin as AF2, initializes TIM3 via the TIM driver at the requested frequency, and sets PWM Mode 1 with duty cycle 0%. pwm_setSignal() recalculates the CCR value from the duty percentage using the cached ARR value:

uint32_t ccr = ((pwm_arr + 1U) * duty_pct) / 100U;

tim_setTimerCompareChannelValue(PWM_TIM, PWM_CHANNEL, ccr);

In the valve simulation, the main loop calls pwm_setSignal(500, 70) to open the valve (70% duty at 500 Hz) and pwm_setSignal(500, 0) to close it, combined with a GPIO LED on PA5 as a visual indicator.

  - Flow Sensor Driver

The YF-S201 sensor outputs a square wave whose frequency is proportional to flow rate. The driver polls the signal pin (PA0) on each main loop iteration and detects rising edges by comparing the current and previous pin states. Each detected rising edge increments a global pulse counter:

if ((state == GPIO_PIN_HIGH) && (last_state == GPIO_PIN_LOW)) {

pulse_count++;

}

Every 1000 ms (measured via SysTick), the main loop reads the accumulated pulse count, resets it, and converts it to liters per minute using the YF-S201 calibration factor of 7.5 pulses per liter per minute:

uint32_t lpm = (pulses * 10U) / 55U;  // scaled integer division

The result is printed over UART and displayed on line 2 of the LCD. When the valve is closed, the pulse counter is reset and the LCD shows '-- L/min' to indicate no active flow.

- I2C Driver and LCD

  - I2C Driver

The Project 1 I2C driver configures I2C1 in Standard Mode at 100 kHz on PB8 (SCL) and PB9 (SDA). Both pins are set to alternate function mode (AF4) with open-drain output type and pull-up resistors enabled through PUPDR. The peripheral is configured with CR2 = 16 (APB1 clock in MHz), CCR = 80 (100 kHz divider), and TRISE = 17 (maximum rise time for standard mode). A software reset (SWRST) is applied before initialization to clear any stuck state from a previous session.

Each I2C transaction follows the sequence: i2c_start() → i2c_sendAddress() → one or more i2c_writeByte() → i2c_stop(). Each step polls the corresponding SR1 flag with a timeout counter to avoid infinite blocking.

  - I2C with Bus Recovery

The Project 2 I2C driver significantly improves robustness with three bug fixes over a previous version. The key addition is i2c_recover(), which handles the case where a slave device (such as the PCF8574) holds SDA LOW after an interrupted transaction, preventing the I2C master from generating START or STOP conditions.

The recovery sequence reconfigures PB8 and PB9 as GPIO open-drain outputs, generates up to 9 manual SCL clock pulses until SDA rises HIGH (freeing the slave), generates a manual STOP condition, performs a software reset of the I2C peripheral, and then calls i2c_init() again to fully reinitialize the peripheral:

for (uint32_t i = 0U; i < 9U; i++) {

if (GPIOB->IDR & (1UL << 9U)) break;  // SDA HIGH — slave released

GPIOB->BSRR = (1UL << (8U + 16U));    // SCL LOW

prv_delay_cycles(500U);

GPIOB->BSRR = (1UL << 8U);            // SCL HIGH

prv_delay_cycles(500U);

}

The prv_check_bus() function is called before every transaction. If SR2.BUSY does not clear within the timeout, i2c_recover() is triggered automatically before attempting the transaction. The prv_stop() function also calls i2c_recover() if the bus remains busy after the STOP condition is issued, preventing the deadlock scenario where every subsequent transaction would see BUSY forever.

  - LCD Driver

The LCD is a standard HD44780 16×2 display connected through a PCF8574 I2C I/O expander at address 0x27. The PCF8574 maps its 8 output pins to the LCD signals as follows: P0=RS, P1=RW, P2=EN, P3=BL (backlight), P4=D4, P5=D5, P6=D6, P7=D7. All communication uses 4-bit mode, meaning each byte is sent as two nibbles — the upper nibble first.

The initialization sequence follows the HD44780 datasheet specification: three 0x03 nibbles are sent to ensure the controller starts in a known state regardless of previous power conditions, then 0x02 to switch to 4-bit mode, followed by the function set (0x28 — 2 lines, 5x8 font), display ON (0x0C — no cursor), clear (0x01), and entry mode (0x06 — increment, no shift).

Project 2 extends the LCD driver with two alternating display pages managed by TIM5. Page 0 shows the system mode (AUTO/MANUAL) and pump state; Page 1 shows the measured distance and water level. Pages alternate every 2 seconds. A mode transition message is shown for 2 seconds when the user switches between AUTO and MANUAL by pressing the button.

To protect the I2C bus from button interrupt interference, the LCD driver in Project 2 disables EXTI0 in the NVIC (via NVIC->ICER) for the approximately 50 µs duration of each individual I2C byte transmission, then re-enables it immediately after. This prevents the button ISR from interrupting a byte mid-transfer, while still allowing the button to be processed between bytes.

- Ultrasonic Sensor and Water Level Classification

  - Measurement Sequence

Each measurement cycle begins with a 5 µs LOW pulse on TRIG (PA9) to pre-stabilize the line, followed by a 20 µs HIGH pulse to trigger the sensor burst, and then LOW again. The driver then polls the ECHO pin (PB3) waiting for it to go HIGH, with a 100 ms timeout to account for weak reflections from water surfaces. Once ECHO goes HIGH, the TIM5 counter is captured as t0. When ECHO returns LOW, t1 is captured and the distance is calculated:

int32_t d = (int32_t)((t1 - t0) * CALIB_SOUND_SPEED / 200000UL)

- (int32_t)CALIB_OFFSET_CM;

The constant CALIB_SOUND_SPEED is set to 3300 (representing 330 m/s × 10 to avoid floating point), and the denominator 200,000 corresponds to 2 × 100,000 µs/s × 10. The factor of 2 accounts for the round-trip travel of the sound wave. Measurements below 25 cm (minimum range) or above 450 cm (maximum range) are discarded.

  - Median Filter and Spike Rejection

Each call to ultrasonic_measure_cm() takes 5 samples (NUM_SAMPLES = 5) with a 100 ms pause between each to allow reverberant echoes to dissipate. Before being added to the sample buffer, each measurement is compared to the previous valid sample. If the difference exceeds 8 cm (SPIKE_THRESHOLD_CM), the sample is flagged as a spike caused by a wall echo or transient reflection and discarded. Valid samples are sorted using a bubble sort and the median value is returned.

If no valid samples are collected in a cycle, the function increments an unknown counter and returns the last known good distance (s_last_valid_cm) rather than zero. Only after 3 consecutive cycles with no valid samples (UNKNOWN_COUNT_MAX = 3) does the function declare a true UNKNOWN state and return 0, signaling to the pump controller to shut off the pump as a safety measure.

  - Water Level Classification

ultrasonic_updateLevel() classifies the returned distance into one of three levels based on the physical dimensions of the water tank used in the prototype. The LED indicators reflect the level state:

- 25–35 cm: WATER_LEVEL_HIGH — tank is full. Green LED on. Pump set to 0% duty.

- 36–40 cm: WATER_LEVEL_MID — tank at medium level. Yellow LED on. Pump set to 75% duty.

- > 40 cm: WATER_LEVEL_LOW — tank is low. Red LED on. Pump set to 100% duty.

- 0 cm (UNKNOWN): No valid reading after 3 consecutive cycles. Red LED on. Pump stopped as safety default.

- Water Pump Control

The pump module wraps the PWM driver to provide a high-level interface keyed to the WaterLevel_t enumeration. pump_init() calls pwm_init() at 1 kHz and starts the timer with 0% duty (pump off). pump_setLevel() maps each water level to a duty cycle:

case WATER_LEVEL_HIGH:   pwm_setSignal(PUMP_DUTY_OFF); break;  // 0%

case WATER_LEVEL_MID:    pwm_setSignal(PUMP_DUTY_MID); break;  // 75%

case WATER_LEVEL_LOW:    pwm_setSignal(PUMP_DUTY_LOW); break;  // 100%

case WATER_LEVEL_UNKNOWN: pwm_setSignal(PUMP_DUTY_OFF); break; // 0% — safe

The pump is connected to PA5 through a 330Ω base resistor driving an NPN transistor (2N2222 or BC547). The STM32 GPIO pin sources approximately 8 mA, which is enough to saturate the transistor base. The collector drives the pump motor, powered from an external 5V supply. A flyback diode is placed in reverse bias across the pump terminals to absorb the inductive spike when the motor is switched off.

In automatic mode, the main loop calls ultrasonic_measure_cm() and ultrasonic_updateLevel() each iteration, then passes the level result to pump_setLevel(). In manual mode, the button ISR sets the pump directly to 100% when toggled ON, and pump_stop() (0%) when toggled OFF, bypassing the sensor logic.

- Push-Button Driver

  - EXTI Configuration

The push-button is connected between PA0 and GND, with the internal pull-up resistor enabled. button_init() configures EXTI0 to trigger on the falling edge (button press) and enables the interrupt in the NVIC at priority level 1. SYSCFG_EXTICR[0] is set to 0x0000 to route EXTI line 0 to GPIOA.

  - Click and Hold Detection

The EXTI0_IRQHandler implements a two-phase detection algorithm. On the first firing (falling edge, button pressed), the ISR verifies pin stability by reading PA0 ten times with 100 µs spacing, stores the TIM5 timestamp in g_t0, and reconfigures EXTI to trigger on the rising edge (release). On the second firing (rising edge, button released), the ISR measures the elapsed duration:

- Duration < 50 ms: treated as mechanical bounce and ignored.

- 50 ms to 3000 ms (click): switches to MANUAL mode and toggles the pump ON or OFF.

- 3000 ms or more (hold): switches back to AUTO mode and resets the pump to off.

After each state change, the g_lcd_dirty flag is set to 1. The main loop checks this flag on every iteration and calls lcd_update() immediately when it is set, ensuring the LCD reflects the new state without waiting for the next sensor measurement cycle to complete.

  - I2C Bus Protection

button_disable() and button_enable() clear and set bit 6 of NVIC->ICER[0] and NVIC->ISER[0] respectively to disable and re-enable the EXTI0 interrupt. In main.c, these calls bracket every lcd_update() call so the button ISR cannot fire during an I2C transaction. Combined with the per-byte NVIC disable inside prv_i2c_write() in lcd.c, this provides two layers of bus protection.

- UART and ADC Drivers

  - UART Driver

USART2 is used for debug output at 115200 baud on PA2 (TX, AF7). uart_init() enables the GPIOA and USART2 clocks, configures PA2 as alternate function 7, sets the baud rate divider (BRR = 16,000,000 / 115,200 ≈ 139), and enables the transmitter (TE) and peripheral (UE) bits in CR1. uart_write() polls the TXE flag in SR before writing each byte to DR. uart_writeString() iterates over null-terminated strings; uart_writeNumber() converts an unsigned integer to ASCII digits and transmits them character by character.

  - ADC Driver

The ADC driver provides an abstraction over ADC1 with support for both regular (single and continuous) and injected conversions. adc_init() enables the RCC clock for ADC1 and GPIOA, configures PA0 in analog mode (MODER = 0x3), and resets CR1 and CR2 to zero (12-bit resolution, software trigger, right-aligned data, single conversion mode by default). The available sample time options range from 3 to 480 cycles, configurable per channel via the SMPR1 and SMPR2 registers. adc_readData() polls the EOC flag and reads the 12-bit result from DR; adc_readInjectedChannelData() waits for JEOC and reads from JDR1 through JDR4 depending on the injected channel index.

- Main Loop Logic

  - Flow Sensor and LCD Main Loop

The main loop is a simple polling loop with three responsibilities. First, it reads the button on PC13 on every iteration to detect a rising edge (button release after pull-down configuration), toggling the valve state variable on each detected press. When the valve is turned ON, the PWM is set to 70% duty and flow counting is enabled; when OFF, PWM goes to 0% and the pulse counter is reset. Second, when flow is enabled, flow_update() is called on every iteration to poll for rising edges on PA0. Third, every 1000 ms (checked via SysTick), the accumulated pulse count is sampled, converted to L/min, transmitted over UART, and written to the LCD row 1.

  - Ultrasonic and Pump Main Loop

The main loop operates in two distinct branches based on the system mode flag managed by the button driver. At the top of each iteration, the loop checks the lcd_dirty flag set by the button ISR and forces an immediate LCD update if it is set, decoupling the display refresh from the sensor measurement cycle.

In AUTO mode, the loop calls ultrasonic_measure_cm() to acquire a filtered distance reading, then ultrasonic_updateLevel() to classify it and control the LED indicators. The resulting level is passed to pump_setLevel() to adjust the PWM duty cycle. The mode is re-checked after the measurement (which takes approximately 500 ms for 5 samples at 100 ms spacing) to prevent race conditions where a button press during measurement would cause the pump to be set to the wrong state. The LCD and UART are updated only if the mode has not changed since the start of the measurement cycle.

In MANUAL mode, the sensor is not polled and the pump is controlled directly by the button ISR. The loop prints the manual state and pump status over UART every 300 ms and updates the LCD accordingly.

- Hardware Pin Summary

The following table consolidates all GPIO pins used across both firmware projects:

| Pin | Signal | Direction | Module / Purpose |
| --- | --- | --- | --- |
| PA0 | FLOW_IN / BTN | Input | Project 1: YF-S201 pulse input (polling). Project 2: Push-button (EXTI0, pull-up, active-LOW) |
| PA2 | USART2_TX | Output | UART debug output to ST-Link VCP at 115200 baud (AF7) |
| PA5 | PUMP_PWM / LED | Output | Project 2: Pump PWM (TIM2_CH1, AF1). Project 1: LED indicator for valve state |
| PA6 | PWM_VALVE | Output | Project 1: PWM valve simulation (TIM3_CH1, AF2, 500 Hz) |
| PA9 | US_TRIG | Output | Project 2: JSN-SR04T TRIG pin — 20 µs pulse to start measurement |
| PB3 | US_ECHO | Input | Project 2: JSN-SR04T ECHO pin — pulse width measured by TIM5 (voltage divider required) |
| PB4 | LED_RED | Output | Project 2: Red LED — WATER_LEVEL_LOW or UNKNOWN |
| PB8 | I2C1_SCL | Output | I2C1 clock line to PCF8574 LCD backpack (AF4, open-drain, pull-up) |
| PB9 | I2C1_SDA | I/O | I2C1 data line to PCF8574 LCD backpack (AF4, open-drain, pull-up) |
| PB10 | LED_YELLOW | Output | Project 2: Yellow LED — WATER_LEVEL_MID |
| PC7 | LED_GREEN | Output | Project 2: Green LED — WATER_LEVEL_HIGH (tank full) |
| PC13 | BTN_VALVE | Input | Project 1: Push-button for valve toggle (pull-down, active-HIGH) |

References

- STM32F411xC/xE Reference Manual (RM0383) — STMicroelectronics. https://www.st.com/resource/en/reference_manual/rm0383-stm32f411xce-advanced-armbased-32bit-mcus-stmicroelectronics.pdf

- STM32 Nucleo-64 User Manual (UM1724) — STMicroelectronics. https://www.st.com/resource/en/user_manual/um1724-stm32-nucleo64-boards-mb1136-stmicroelectronics.pdf

- HD44780 LCD Controller Datasheet — Hitachi Semiconductor.

- PCF8574 Remote 8-Bit I/O Expander Datasheet — Texas Instruments. https://www.ti.com/product/PCF8574

- JSN-SR04T Waterproof Ultrasonic Distance Sensor Datasheet.

- YF-S201 Water Flow Sensor Datasheet. https://domoticx.net/docs/yf-s201-information/