CENTRO DE ENSEÑANZA TÉCNICA Y SUPERIOR

Section Priority Control via Sensor Testing

Automated Irrigation System

Teacher: Carlos Alberto Villareal Molina

Oskar Liborio García Velíz — 38265

Carlos Humberto Araiza Quintana — 36986

Luis Angel Lugo Muñiz — 37915

Sebastian Lizalde Herrera — 38241

Tijuana, B.C. — May 24th, 2026

# Section Priority Control via Sensor Testing

This report documents the firmware progress for the Automated Irrigation System developed on the STM32F411RE using bare-metal embedded C in Visual Studio Code. The focus of this phase is the independent testing and validation of every sensor component that drives section-level priority control. The system manages two irrigation sections (A and B), each equipped with a YL-69 soil moisture sensor and a MOT-135 servo motor acting as a physical valve actuator. A separate subsystem using the JSN-SR04T ultrasonic sensor monitors the water tank level and controls a refill pump. The G1/4 water flow sensor has been independently validated and will be integrated after physical valve coupling is complete.

# Software Architecture and Module Overview

The firmware is organized in a layered driver architecture. Low-level peripheral drivers expose clean APIs consumed by higher-level functional modules, keeping hardware-specific logic isolated from application logic.

## Irrigation Sections — Module Breakdown

| Module / File | Responsibility |
| --- | --- |
| soil_moisture.c / .h | YL-69 driver: ADC1 CH10 (PC0 – Section A) and CH11 (PC1 – Section B). Reads raw ADC counts, applies inverse linear mapping to percentage (0–100%), and classifies moisture level (DRY / OK / WET). |
| irrigation.c / .h | Hysteresis state machine (IDLE → FULL → HALF → CLOSED). Reads both soil sensors, evaluates thresholds, and dispatches servo commands and LED indicators per section independently. |
| servo.c / .h | PWM servo control via TIM3: CH1 on PC6 (Section A) and CH4 on PC9 (Section B). 50 Hz signal, CCR maps 0°→1000 µs, 90°→1500 µs, 180°→2000 µs. Both channels initialized and enabled simultaneously. |
| adc_driver.c / .h | ADC1 bare-metal driver: single conversion mode, configurable channel and sample time (480 cycles). Called by soil_moisture.c for CH10 and CH11 sequentially. |
| gpio_driver.c / .h | GPIO abstraction using GPIO_PinCfg_t struct: mode, output type, speed, pull resistor. Supports analog, output, input, and alternate function modes. Atomic BSRR writes for set/clear. |
| tim_driver.c / .h | Timer abstraction for TIM1–TIM5, TIM9–TIM11. Provides frequency-based and ms-period configuration, compare mode (PWM Mode 1), channel enable, and ARR preload. |
| i2c_driver.c / .h | I2C1 driver (PB8 SCL, PB9 SDA, 100 kHz). Includes bus recovery routine (i2c_recover) generating up to 9 manual SCL pulses to free a stuck SDA line. Error flag handling and prv_check_bus() before each transaction. |
| lcd.c / .h | HD44780 16x2 via PCF8574 I2C backpack at address 0x27. 4-bit mode. Displays moisture percentage and valve state (ABT 180 / MED 90 / CRR 0 / ESPERA) for both sections simultaneously. |
| timer.c / .h | TIM4-based blocking delay module (timer_delay_ms). Used by the main loop to set the 5-second irrigation polling period. |

## Water Tank Subsystem — Module Breakdown

| Module / File | Responsibility |
| --- | --- |
| ultrasonic.c / .h | JSN-SR04T driver: TIM5 @1 MHz free-running counter (PSC=15, ARR=0xFFFFFFFF). Takes 5 samples per cycle with 100 ms inter-sample delay. Applies spike rejection filter (±8 cm threshold) and bubble-sort median. Returns last valid reading on transient failures; declares UNKNOWN only after 3 consecutive failed cycles. |
| pump.c / .h | Pump PWM driver wrapping the PWM module. Maps WaterLevel_t to 0%, 75%, or 100% duty cycle on TIM2_CH1 (PA5). Activates refill pump when tank level drops below threshold. |
| flowsensor.c / .h | G1/4 water flow sensor driver (PA0, pull-down). Polls for rising edges each main loop iteration and accumulates pulse count. Converts to L/min using the YF-S201 calibration factor (7.5 pulses/L/min) every 1000 ms via SysTick. Validated independently — integration pending physical valve coupling. |
| pwm.c / .h | PWM wrapper for TIM3_CH1 on PA6 (AF2). Used in the flow sensor test bench to simulate valve control at 500 Hz, 70% duty cycle when valve is open and 0% when closed. |
| uart.c / .h | USART2 TX driver at 115200 baud on PA2 (AF7). Used in the flow sensor test for serial debug output: valve state, pulse count, and calculated flow rate in L/min. |

# Soil Moisture Sensor Module (YL-69)

The soil_moisture module provides the complete sensing layer for both irrigation sections. Each YL-69 sensor is a resistive probe: dry soil produces high resistance, high voltage, and a high ADC count; wet soil produces the inverse. Both sensors are read sequentially through ADC1 using single-conversion mode.

## ADC Configuration and Pin Assignment

| Section | GPIO Pin | ADC Channel | Sample Time |
| --- | --- | --- | --- |
| A | PC0 | ADC1_CH10 | 480 cycles |
| B | PC1 | ADC1_CH11 | 480 cycles |

Both pins are configured in analog mode (MODER = 0x3, no pull resistor) through the GPIO_PinCfg_t struct. The ADC reads each channel sequentially by calling adc_setChannel() followed by adc_startSingleConversion() and adc_readData().

## Percentage Conversion — Inverse Linear Mapping

The YL-69 output is inverted with respect to moisture: higher ADC counts correspond to dryer soil. The driver applies the following inverse linear mapping calibrated to measured endpoints:

RAW_DRY = 3200   (air — fully dry)

RAW_WET =  800   (fully submerged in water)

percent = ((RAW_DRY - raw) × 100) / (RAW_DRY - RAW_WET)

Values at or beyond the calibration endpoints are clamped: raw ≥ RAW_DRY → 0%, raw ≤ RAW_WET → 100%.

## Moisture Level Classification

| Level | Threshold | Meaning |
| --- | --- | --- |
| SOIL_LEVEL_DRY | < 30% | Soil is dry — triggers irrigation cycle |
| SOIL_LEVEL_OK | 30% – 80% | Normal moisture range — hysteresis zone |
| SOIL_LEVEL_WET | > 80% | Soil saturated — irrigation cycle ends |

## Sensor Validation Results

Both sensors were tested across three physical conditions before integration into the irrigation state machine:

| Condition | Section A | Section B | LED Status |
| --- | --- | --- | --- |
| Sensor in air (DRY) | 0% | 0% | LED A (PA5) and LED B (PA6) OFF |
| Sensor in water (WET) | 100% | 100% | Both LEDs ON |
| Sensor in wet soil (OK) | 90%+ | 90%+ | Both LEDs ON — valve logic active |

Both sections operate fully independently, confirming that ADC channel switching between CH10 and CH11 does not cause cross-contamination between readings.

# Irrigation Control — Hysteresis State Machine

The irrigation module implements a four-state machine with hysteresis for each section independently. The hysteresis design prevents oscillation around threshold boundaries by requiring moisture to exceed a higher value to transition upward and drop below a lower value to restart the cycle.

## State Definitions and Transitions

| State | Servo Angle | Moisture Range | Transition Condition |
| --- | --- | --- | --- |
| VALVE_IDLE | 0° | Any | Moisture < 30% → VALVE_FULL |
| VALVE_FULL | 180° | 0% – 59% | Moisture ≥ 60% → VALVE_HALF |
| VALVE_HALF | 90° | 60% – 79% | Moisture ≥ 80% → VALVE_CLOSED / < 30% → VALVE_FULL |
| VALVE_CLOSED | 0° | > 80% | Moisture < 30% → VALVE_FULL (cycle restarts) |

The state machine is evaluated every 5 seconds (IRRIGATION_PERIOD_MS = 5000). State transitions only apply servo and LED changes when the next state differs from the current one, avoiding redundant PWM writes. The main loop structure is:

while (1) {

irrigation_update();       // reads both sensors, updates state machine

timer_delay_ms(5000);       // 5-second polling period via TIM4

}

## LCD Display Format

The LCD 16x2 display shows real-time moisture and valve state for both sections simultaneously, one section per row:

Row 0: SecA: 45% MED  90

Row 1: SecB: 12% ABT 180

| LCD Tag | Meaning |
| --- | --- |
| ABT 180 | Valve fully open — servo at 180° (VALVE_FULL state) |
| MED 90 | Valve half-open — servo at 90° (VALVE_HALF state) |
| CRR 0 | Valve fully closed — servo at 0° (VALVE_CLOSED state) |
| ESPERA | Idle — waiting for moisture to drop below 30% (VALVE_IDLE state) |

# Servo Motor Control Module

The servo module generates the 50 Hz PWM signal required by standard RC servo motors (MOT-135, 180°) using TIM3 with two independent output channels. Both servos are initialized simultaneously and can be commanded to any angle between 0° and 180° through a linear CCR mapping.

## PWM Configuration

| Parameter | Value | Description |
| --- | --- | --- |
| Timer | TIM3 | APB1 timer, supports CH1–CH4 |
| Frequency | 50 Hz | Standard servo PWM period = 20 ms |
| PSC | 15 | 16 MHz / 16 = 1 MHz → 1 tick = 1 µs |
| ARR | 19999 | Period = 20 000 µs = 20 ms |
| CH1 / PC6 | AF2 (TIM3) | Section A servo — PWM Mode 1 |
| CH4 / PC9 | AF2 (TIM3) | Section B servo — PWM Mode 1 |

## Angle-to-CCR Mapping

The servo angle is converted to a CCR value using a linear interpolation between the calibrated 0° and 180° pulse widths:

CCR = SERVO_CCR_0DEG + (angle × (SERVO_CCR_180DEG − SERVO_CCR_0DEG)) / 180

| Angle | CCR (µs) | Valve State |
| --- | --- | --- |
| 0° | 1000 | Fully closed — VALVE_CLOSED / VALVE_IDLE |
| 90° | 1500 | Half-open — VALVE_HALF |
| 180° | 2000 | Fully open — VALVE_FULL |

## Validation Results

- PWM signal at 50 Hz confirmed on Section A servo (PC6 / TIM3_CH1).

- Physical movement from 0° to 180° validated on Section A.

- Section B servo (PC9 / TIM3_CH4) code path validated — full physical test pending external power supply, as USB current is insufficient to drive both servos simultaneously.

- Both channels are enabled via tim_enableTimerCompareChannel() and share the same TIM3 counter, ensuring synchronous 50 Hz PWM generation.

# Water Tank Subsystem — JSN-SR04T Ultrasonic Sensor

The water tank monitoring subsystem is a separate, independent module developed in parallel with the irrigation section control. The JSN-SR04T waterproof ultrasonic sensor measures the distance from the sensor to the water surface inside the storage tank. When the level drops below a defined threshold, the system activates the refill pump automatically.

## Measurement Sequence

Each measurement cycle begins with a 5 µs LOW pre-stabilization pulse on TRIG (PA9), followed by a 20 µs HIGH trigger pulse. The driver then polls the ECHO pin (PB3) for the rising edge with a 100 ms timeout, captures the TIM5 counter at t0, and measures the falling edge at t1:

distance_cm = ((t1 - t0) × CALIB_SOUND_SPEED / 200 000) − CALIB_OFFSET_CM

The 100 ms timeout (increased from 50 ms) accommodates weak reflections from water surfaces that produce longer ECHO delays. Measurements below 25 cm (minimum sensor range) or above 450 cm are discarded.

## Median Filter and Spike Rejection

Each call to ultrasonic_measure_cm() takes 5 samples with 100 ms inter-sample pauses. The algorithm applies two filtering layers:

- Spike rejection: samples differing more than 8 cm from the previous valid sample are discarded as wall echoes or transient reflections.

- Median filter: valid samples are sorted via bubble sort and the median is returned, eliminating outliers from the remaining set.

- Graceful degradation: if no valid samples are collected in a cycle, the last known good distance is returned. Only after 3 consecutive failed cycles (UNKNOWN_COUNT_MAX = 3) does the function declare a true UNKNOWN state and return 0, which triggers the pump safety shutdown.

## Water Level Classification and LED Indicators

| Level | Distance Range | LED | Pump Duty |
| --- | --- | --- | --- |
| WATER_LEVEL_HIGH | 25 – 35 cm | Green (PC7) | 0% — tank full, pump off |
| WATER_LEVEL_MID | 36 – 40 cm | Yellow (PB10) | 75% — tank at medium level |
| WATER_LEVEL_LOW | > 40 cm | Red (PB4) | 100% — tank low, pump at full speed |
| WATER_LEVEL_UNKNOWN | 0 (3 cycles) | Red (PB4) | 0% — no signal, pump off (safe) |

# Water Flow Sensor — G1/4 (Independent Validation)

The G1/4 water flow sensor was tested and validated in an independent test bench prior to system integration. The flowsensor module polls the sensor signal pin on rising edges each main loop iteration and accumulates a pulse count. Every 1000 ms (measured via SysTick), the accumulated pulses are converted to a flow rate in L/min.

## Driver Implementation

The sensor signal is connected to PA0 with an internal pull-down resistor. The driver detects rising edges by comparing the current and previous pin states on each loop iteration:

if ((state == GPIO_PIN_HIGH) && (last_state == GPIO_PIN_LOW)) {

pulse_count++;

}

Every 1000 ms the main loop reads and resets the counter, then converts pulses to liters per minute using the sensor's calibration factor:

lpm = (pulse_count × 10) / 55        // integer scaled division

The calibration constant 55 corresponds to the YF-S201 factor of 7.5 pulses per L/min scaled by 10 to avoid floating-point arithmetic in the integer path. The float version using FLOW_PULSES_PER_LITER_MIN = 7.5 is also provided in flow_getLitersPerMin() for higher-precision contexts.

## Test Bench Configuration

| Signal | Pin | Function |
| --- | --- | --- |
| Flow pulse | PA0 | Sensor signal input — pull-down, rising edge detection |
| Valve PWM | PA6 (TIM3_CH1) | Simulates valve open (500 Hz, 70% duty) / closed (0%) |
| LED | PA5 | Visual indicator — ON when valve is open |
| Button | PC13 | Toggle valve ON/OFF — pull-down, rising edge |
| UART debug | PA2 (USART2) | 115200 baud — prints flow rate and valve state every second |
| LCD output | PB8/PB9 (I2C1) | Displays valve state and flow rate in L/min in real time |

Validation confirmed that: the LCD updates correctly when the valve is toggled ON and OFF; the UART prints a new flow reading every second while the valve is open; and the pulse counter resets cleanly on valve close, displaying '-- L/min' when no flow is present.

Integration into the main irrigation firmware will proceed after the servo motors are physically coupled to the water valve handles, allowing the G1/4 sensor to measure actual water flow per section.

# Hardware Pin Summary

| Pin | Signal | Direction | Module / Purpose |
| --- | --- | --- | --- |
| PC0 | SOIL_A | Input (Analog) | YL-69 Section A — ADC1_CH10, soil moisture reading |
| PC1 | SOIL_B | Input (Analog) | YL-69 Section B — ADC1_CH11, soil moisture reading |
| PC6 | SERVO_A_PWM | Output | Section A servo — TIM3_CH1, AF2, 50 Hz PWM |
| PC9 | SERVO_B_PWM | Output | Section B servo — TIM3_CH4, AF2, 50 Hz PWM |
| PA5 | LED_A | Output | Section A moisture indicator LED |
| PA6 | LED_B / PWM | Output | Section B indicator LED / Flow test valve PWM |
| PA0 | FLOW_IN | Input | G1/4 flow sensor pulse input — pull-down, polling |
| PA2 | USART2_TX | Output | UART debug — 115200 baud, AF7 |
| PA9 | US_TRIG | Output | JSN-SR04T trigger — 20 µs pulse |
| PB3 | US_ECHO | Input | JSN-SR04T echo — voltage divider required (10k/20k) |
| PB4 | LED_RED | Output | Tank level indicator — WATER_LEVEL_LOW or UNKNOWN |
| PB8 | I2C1_SCL | Output | I2C1 clock — PCF8574 LCD backpack, AF4 open-drain |
| PB9 | I2C1_SDA | I/O | I2C1 data — PCF8574 LCD backpack, AF4 open-drain |
| PB10 | LED_YELLOW | Output | Tank level indicator — WATER_LEVEL_MID |
| PC7 | LED_GREEN | Output | Tank level indicator — WATER_LEVEL_HIGH (full) |
| PC13 | BTN_FLOW | Input | Flow sensor test button — valve toggle, pull-down |

# Pending Tasks and Next Steps

## Hardware Integration

- Physical coupling of Section A and Section B servo motors to the water valve handles.

- External power supply integration for both servos to overcome USB current limitations (NUCLEO-F411RE USB limit ~500 mA is insufficient for dual servo operation).

- Complete physical validation of Section B servo movement from 0° to 180° with external power.

## Software Integration

- Integration of the G1/4 water flow sensor module into the main irrigation firmware after physical valve coupling, to measure actual water flow per section when the servo opens the valve.

- Full system integration combining irrigation section control, water flow monitoring per section, and the tank level subsystem into a single unified firmware.

- End-to-end closed-loop test: soil moisture sensor triggers valve open → flow sensor confirms water delivery → moisture rises to threshold → valve closes automatically.

# References

- STM32F411xC/xE Reference Manual (RM0383) — STMicroelectronics. https://www.st.com/resource/en/reference_manual/rm0383-stm32f411xce-advanced-armbased-32bit-mcus-stmicroelectronics.pdf

- STM32 Nucleo-64 User Manual (UM1724) — STMicroelectronics. https://www.st.com/resource/en/user_manual/um1724-stm32-nucleo64-boards-mb1136-stmicroelectronics.pdf

- PCF8574 Remote 8-Bit I/O Expander Datasheet — Texas Instruments. https://www.ti.com/product/PCF8574

- JSN-SR04T Waterproof Ultrasonic Distance Sensor Datasheet.

- YL-69 / HW-103 Soil Moisture Sensor Module Datasheet.

- YF-S201 / G1-4 Water Flow Sensor Datasheet. https://domoticx.net/docs/yf-s201-information/