CENTRO DE ENSEÑANZA TÉCNICA Y SUPERIOR

Implementation of Valve Control, Water Pump, and Relay Module in the STM32

Automated Irrigation System

Teacher: Carlos Alberto Villareal Molina

Oskar Liborio García Velíz — 38265

Carlos Humberto Araiza Quintana — 36986

Luis Angel Lugo Muñiz — 37915

Sebastian Lizalde Herrera — 38241

Tijuana, B.C. — May 16th, 2026

Implementation of Valve Control, Water Pump, and Relay Module in the STM32

This document describes how the solenoid valves, the water pump, and the relay module (ARD-345) are integrated as digital outputs of the STM32F411RE microcontroller in the automated irrigation system. The system controls two independent irrigation sections, each with its own solenoid valve, plus a single shared water pump. All three actuators are devices driven by GPIO output pins on the STM32. Because these actuators operate at voltages and currents far beyond the STM32 GPIO safe limits, the relay module serves as the power interface between the 3.3 V microcontroller logic and the 12 V valve / pump circuits.

- Relay Module ARD-345

  - Role in the System

The relay module is the central power interface between the STM32F411RE and the high-power actuators. The STM32 GPIO pins operate at 3.3 V logic and can only source or sink a few milliamps of current, far below what is needed to directly switch solenoid valves or the water pump. The ARD-345 relay module accepts the low-current 3.3 V control signal from the STM32 and uses it to switch an independent high-current circuit that powers the actuators.

The module includes an internal optocoupler that electrically isolates the STM32's 3.3 V logic side from the 12 V actuator side, protecting the microcontroller from voltage transients generated when inductive loads (solenoid coils, pump motor) are switched off. A single relay module with multiple channels is used in this project: one channel per solenoid valve (Section A and Section B) and one channel for the water pump.

  - Electrical Interface with the STM32

The relay module's VCC pin is powered from the Nucleo-64's 5V header pin (CN6 pin 5), and its GND is shared with the STM32 ground. Each IN control pin is driven directly from a 3.3 V STM32 GPIO output pin. Most 5V relay modules use active-LOW logic: pulling IN LOW energizes the relay coil and closes the circuit to the load; pulling IN HIGH de-energizes the relay and opens the circuit. This means the STM32 must write a logical 0 to the GPIO pin to activate an actuator and a logical 1 to deactivate it. The firmware must initialize all relay control pins HIGH at startup to prevent unintended actuation during system boot.

  - STM32 Pin Assignment

The three relay channels are controlled by the following GPIO pins:

| Parameter | Value |
| --- | --- |
| PB6 | Relay channel → Solenoid valve Section A |
| PC7 | Relay channel → Solenoid valve Section B |
| PA8 | Relay channel → Water pump |

  - Bare Metal GPIO Configuration

The three control pins must be configured as GPIO outputs. The following steps show the register-level configuration:

Step 1 — Enable GPIO clocks:

RCC->AHB1ENR |= RCC_AHB1ENR_GPIOBEN;  // Enable GPIOB clock (PB6)

RCC->AHB1ENR |= RCC_AHB1ENR_GPIOCEN;  // Enable GPIOC clock (PC7)

RCC->AHB1ENR |= RCC_AHB1ENR_GPIOAEN;  // Enable GPIOA clock (PA8)

Step 2 — Set pins as General Purpose Output (push-pull):

// PB6 → Output (MODER bits [13:12] = 01)

GPIOB->MODER |=  (1U << 12);

GPIOB->MODER &= ~(1U << 13);

// PC7 → Output (MODER bits [15:14] = 01)

GPIOC->MODER |=  (1U << 14);

GPIOC->MODER &= ~(1U << 15);

// PA8 → Output (MODER bits [17:16] = 01)

GPIOA->MODER |=  (1U << 16);

GPIOA->MODER &= ~(1U << 17);

Step 3 — Configure as push-pull, no pull resistors:

GPIOB->OTYPER &= ~(1U << 6);   // PB6 push-pull

GPIOC->OTYPER &= ~(1U << 7);   // PC7 push-pull

GPIOA->OTYPER &= ~(1U << 8);   // PA8 push-pull

GPIOB->PUPDR  &= ~(3U << 12);  // No pull on PB6

GPIOC->PUPDR  &= ~(3U << 14);  // No pull on PC7

GPIOA->PUPDR  &= ~(3U << 16);  // No pull on PA8

Step 4 — Initialize all relay pins HIGH at startup: This ensures all relays start de-energized (active-LOW logic), preventing unintended actuation during system initialization.

GPIOB->ODR |= (1U << 6);   // PB6 HIGH → Valve A relay OFF

GPIOC->ODR |= (1U << 7);   // PC7 HIGH → Valve B relay OFF

GPIOA->ODR |= (1U << 8);   // PA8 HIGH → Pump relay OFF

  - Key Specifications

| Parameter | Value |
| --- | --- |
| Module model | ARD-345 (single-channel, 5V relay) |
| Operating voltage | 5 V DC (coil) |
| Control signal | 3.3 V / 5 V logic compatible |
| Control input current | ~0.95 mA from STM32 GPIO (at 3.3 V) |
| Output terminals | COM, NO, NC |
| Isolation | Optocoupler (STM32 electrically isolated from load) |
| Trigger logic | Active-LOW (IN = LOW → relay energized) |
| STM32 control pins | PB6 (Valve A), PC7 (Valve B), PA8 (Pump) |

  - Important Considerations

- Always use a 5V relay module, not a 12V one. A 12V relay module has a higher internal resistance that reduces optocoupler drive current below the reliable switching threshold when driven by a 3.3V GPIO signal.

- Inductive loads such as solenoid valves and pump motors generate voltage spikes when switched off. The relay module's optocoupler provides the primary protection, but a flyback diode (1N4001) placed in reverse bias across each actuator's terminals is strongly recommended as a secondary measure.

- The relay module VCC must be connected to the Nucleo-64's 5V pin, not the 3.3V rail. The relay coil itself requires 5V to operate; only the IN control signal is at 3.3V.

- All relay control pins must be driven HIGH before any other initialization code runs to prevent the actuators from briefly activating during the GPIO configuration sequence.

- Solenoid Valves FPD-270A

  - Operating Principle

The FPD-270A is a normally-closed (NC), pilot-operated 12V DC solenoid valve. One valve is placed inline with each irrigation section pipe. Being normally closed means that when no electrical power is applied to the valve coil, the internal spring-loaded plunger blocks water flow — a safe default state for an irrigation system, since a power failure or reset will automatically stop all irrigation without requiring any software action.

When the relay energizes and applies 12V DC to the solenoid coil, the electromagnetic field pulls the plunger back against the spring, opening the valve and allowing water to flow through the pipe. When the relay de-energizes, the spring returns the plunger to its closed position, stopping flow. This ON/OFF behavior is entirely controlled by the relay, which in turn is controlled by the STM32 GPIO output.

The valve is pilot-operated, meaning it requires a minimum inlet water pressure of 0.02 MPa (approximately 0.2 bar) to fully close. If the system operates below this pressure, the valve may not seal completely and could allow a small amount of water to pass even when nominally closed.

  - Control Logic from the STM32

From the STM32's perspective, controlling a solenoid valve is simply a matter of writing a HIGH or LOW value to the corresponding GPIO output pin. The relay module inverts this signal (active-LOW), so the mapping is:

| STM32 GPIO | Relay IN pin | Relay state | Valve state |
| --- | --- | --- | --- |
| HIGH (1) | HIGH | De-energized | CLOSED (no flow) |
| LOW (0) | LOW | Energized | OPEN (flow) |

The activation and deactivation of each valve is performed by writing to the ODR (Output Data Register) of the corresponding GPIO port:

Activate Valve A (Section A — open):

GPIOB->ODR &= ~(1U << 6);  // PB6 LOW → relay energized → valve opens

Deactivate Valve A (Section A — close):

GPIOB->ODR |=  (1U << 6);  // PB6 HIGH → relay de-energized → valve closes

Activate Valve B (Section B — open):

GPIOC->ODR &= ~(1U << 7);  // PC7 LOW → relay energized → valve opens

Deactivate Valve B (Section B — close):

GPIOC->ODR |=  (1U << 7);  // PC7 HIGH → relay de-energized → valve closes

  - Irrigation Sequence Logic

In the automated irrigation system, each section is irrigated independently based on the soil moisture reading from its corresponding YL-69 sensor. The general decision logic for each section is the following:

if (soil_moisture_A < MOISTURE_THRESHOLD) {

// Soil is too dry — open valve and start pump

GPIOB->ODR &= ~(1U << 6);   // Open Valve A

GPIOA->ODR &= ~(1U << 8);   // Start pump

} else {

// Soil moisture is sufficient — close valve

GPIOB->ODR |=  (1U << 6);   // Close Valve A

}

The same logic applies independently to Section B using PC7 and the SOIL_B ADC reading. Only one section is irrigated at a time to avoid exceeding the pump's flow capacity. If both sections require irrigation simultaneously, a priority scheme is implemented: Section A is irrigated first until its moisture reaches the threshold, then Section B.

  - Key Specifications

| Parameter | Value |
| --- | --- |
| Model | FPD-270A |
| Working voltage | DC 12 V |
| Current | 400 mA ¦ Power: 4.8 W |
| Flow rate range | 0.5 – 15 L/min |
| Working pressure | 0.02 – 0.8 MPa |
| Type | Normally closed, pilot-operated |
| Fluid | Tap water, 0 – 55 °C |
| Durability | 50,000 cycles |
| Dimensions | 68 × 34 × 64 mm |

  - Important Considerations

- Water flows in one direction only through the FPD-270A. The inlet must be correctly oriented according to the arrow marked on the valve body; reversed installation will prevent the valve from operating correctly.

- A minimum inlet pressure of 0.02 MPa is required for complete valve closure. Verify that the water supply system maintains this minimum pressure even when the pump is off.

- Always include a flyback diode (1N4001) in reverse bias across the solenoid coil terminals. Without it, the voltage spike generated when the coil de-energizes can damage the relay contacts over time.

- Do not switch the valve at a high frequency. Each actuation counts toward the 50,000-cycle durability rating. The irrigation logic should avoid oscillating the valve rapidly when the soil moisture is near the threshold a hysteresis band should be implemented in the control logic.

- Water Pump

  - Operating Principle

The water pump used in this project is a small DC submersible pump that moves water from the supply tank through the irrigation pipes and to the active section. It operates with a low-voltage DC motor that spins an internal impeller, generating the flow required to feed the solenoid valves. The pump is always powered through the relay module, never directly from the STM32 GPIO, since its current draw (130–220 mA) far exceeds the safe output of any microcontroller pin.

The pump has a single ON/OFF operating mode: when the relay energizes and connects the pump to its external DC supply, the motor spins and generates flow. When the relay de-energizes, the motor stops immediately. There is no speed control in this system the pump always runs at full power when active.

  - Control Logic from the STM32

The pump is controlled through GPIO pin PA8, which drives the third channel of the relay module. The same active-LOW logic as the valves applies:

Start the pump:

GPIOA->ODR &= ~(1U << 8);  // PA8 LOW → relay energized → pump ON

Stop the pump:

GPIOA->ODR |=  (1U << 8);  // PA8 HIGH → relay de-energized → pump OFF

  - Pump and Valve Coordination

The pump and the solenoid valves must always be activated in the correct sequence to protect both the pump and the valve. The recommended sequence for starting and stopping an irrigation cycle is:

Irrigation start sequence: Open the target valve first, then start the pump. Starting the pump before the valve opens would force it to run against a closed circuit, which generates excessive back-pressure and can damage the pump impeller.

// Correct start sequence — Valve before Pump

GPIOB->ODR &= ~(1U << 6);   // 1. Open Valve A

delay_ms(200);               // 2. Allow valve to fully open

GPIOA->ODR &= ~(1U << 8);   // 3. Start pump

Irrigation stop sequence: Stop the pump first, then close the valve. Closing the valve while the pump is still running creates a water hammer effect — a pressure spike that can damage both the valve and the pipe connections.

// Correct stop sequence — Pump before Valve

GPIOA->ODR |=  (1U << 8);   // 1. Stop pump

delay_ms(200);               // 2. Allow flow to stop

GPIOB->ODR |=  (1U << 6);   // 3. Close Valve A

  - Safety Conditions

The firmware must enforce two additional safety conditions before activating the pump:

Water level check: The JSN-SR04T sensor reading is checked before every irrigation cycle. If the measured water level in the supply tank falls below a defined minimum threshold, the pump must not be activated, as running the pump dry will damage its motor. The tank level check is performed as follows:

if (water_level_cm >= TANK_MIN_LEVEL_CM) {

// Safe to start pump

GPIOB->ODR &= ~(1U << 6);  // Open valve

delay_ms(200);

GPIOA->ODR &= ~(1U << 8);  // Start pump

} else {

// Tank too low — skip irrigation cycle

}

Valve open before pump: The firmware must verify that the target section valve has been commanded open before activating the pump, as described in the start sequence above. A simple flag variable per section tracks the current valve state.

  - Key Specifications

| Parameter | Value |
| --- | --- |
| Source | Steren DC mini water pump |
| Operating voltage | 2.5 – 6 V DC |
| Current | 130 – 220 mA |
| Power consumption | 0.4 – 1.5 W |
| Flow rate | 80 – 120 L/h |
| Maximum lift height | 1 m |

  - Important Considerations

- Never run the pump without water in the tank (dry running). The pump motor relies on water for cooling and lubrication; dry operation will damage or destroy the impeller within seconds.

- The pump must be powered from an external 5V DC supply, not from the Nucleo-64 board. The board's 5V pin can supply limited current and powering the pump from it would cause voltage drops that affect the entire system.

- Always follow the valve-before-pump start sequence and the pump-before-valve stop sequence to avoid water hammer and pump over-pressure damage.

- If a transistor driver (MOSFET) is used instead of the relay module to switch the pump, a flyback diode must be placed across the pump motor terminals to absorb the inductive spike when the motor is switched off.

- Complete Output Activation Summary

The following table summarizes all GPIO output pins, their associated actuators, and the logic levels required to activate and deactivate each one:

| Pin | Signal name | Actuator | Active (ON) | Inactive (OFF) |
| --- | --- | --- | --- | --- |
| PB6 | RELAY_VALVE_A | Solenoid Valve A | GPIO LOW (ODR &= ~bit) | GPIO HIGH (ODR ¦= bit) |
| PC7 | RELAY_VALVE_B | Solenoid Valve B | GPIO LOW (ODR &= ~bit) | GPIO HIGH (ODR ¦= bit) |
| PA8 | RELAY_BOMBA | Water Pump | GPIO LOW (ODR &= ~bit) | GPIO HIGH (ODR ¦= bit) |

All three pins share the same Bare Metal activation and deactivation pattern. The irrigation control logic in the main loop evaluates sensor readings, enforces safety conditions (water level, valve-pump sequence), and writes to the corresponding ODR bits to command each actuator.

References

- STM32F411xC/xE Reference Manual (RM0383) — STMicroelectronics. https://www.st.com/resource/en/reference_manual/rm0383-stm32f411xce-advanced-armbased-32bit-mcus-stmicroelectronics.pdf

- Controlling a Solenoid Valve with Arduino — BC Robotics. https://bc-robotics.com/tutorials/controlling-a-solenoid-valve-with-arduino/

- Driving Relays with STM32 Microcontroller — Engineers Garage. https://www.engineersgarage.com/driving-relays-with-stm32-microcontroller/

- Can a 3.3V Signal Reliably Switch a 5V Relay Module? — PT Solutions. https://ptsolns.com/blogs/tinker-thoughts/ttb8-can-a-3-3v-signal-reliably-switch-a-5v-relay-module

- 5V Single Channel Relay Module — Components101. https://components101.com/switches/5v-single-channel-relay-module-pinout-features-applications-working-datasheet