CENTRO DE ENSEÑANZA TÉCNICA Y SUPERIOR

Soil Moisture, Temperature, and Water Level Sensor Integration in the STM32

Automated Irrigation System

Teacher: Carlos Alberto Villareal Molina

Oskar Liborio García Velíz — 38265

Carlos Humberto Araiza Quintana — 36986

Luis Angel Lugo Muñiz — 37915

Sebastian Lizalde Herrera — 38241

Tijuana, B.C. — May 16th, 2026

Soil Moisture, Temperature, and Water Level Sensor Integration in the STM32

This document describes how the soil moisture sensor (YL-69), the temperature and humidity sensor (DHT11), and the water level sensor (JSN-SR04T) are integrated as inputs into the STM32F411RE microcontroller. All peripheral configuration and data acquisition is implemented using Bare Metal embedded programming — directly accessing the STM32 registers through the CMSIS headers without any Hardware Abstraction Layer (HAL) library.

Each sensor section covers its operating principle, the STM32 peripheral it uses, the register-level configuration required, and the data acquisition logic.

- Soil Moisture Sensor YL-69

Operating Principle

The YL-69 is a resistive soil moisture sensor consisting of two components: the probe (two conductive pads inserted into the soil) and a control board with an LM393 voltage comparator. The two probes act as a variable resistor in the soil — when the soil is wet, its electrical conductivity increases and the resistance between the probes decreases, causing the analog output voltage to drop. Conversely, when the soil is dry, resistance increases and the output voltage rises.

The module provides two output modes. The analog output (AO) delivers a voltage between 0 V and VCC proportional to the measured resistance, which is connected to an ADC channel on the STM32. The digital output (DO) produces a binary HIGH/LOW signal from the internal LM393 comparator, whose switching threshold is set via the on-board potentiometer. In this project, the AO pin is used to provide precise, continuous moisture readings to the microcontroller.

1.2 STM32 Peripheral: ADC1

The YL-69 analog output is read by the STM32F411RE using ADC1. The two sensors are connected to pins PC0 (ADC1_CH10) and PC1 (ADC1_CH11), accessible via the morpho connector CN7 of the Nucleo-64 board.

The following register-level steps configure ADC1 in single conversion mode for Bare Metal operation:

Step 1 — Enable clocks: The GPIO port C clock and the ADC1 clock must be enabled through the RCC registers before any peripheral configuration.

RCC->AHB1ENR |= RCC_AHB1ENR_GPIOCEN;    // Enable GPIOC clock

RCC->APB2ENR |= RCC_APB2ENR_ADC1EN;     // Enable ADC1 clock

Step 2 — Configure GPIO pins as analog: PC0 and PC1 must be set to Analog mode in their MODER register. No pull-up or pull-down resistors are needed.

GPIOC->MODER |= (3U << 0) | (3U << 2);  // PC0 and PC1 → Analog mode

Step 3 — Configure ADC1: Set 12-bit resolution, single conversion mode, and right data alignment. Select the channel to convert in the ADC regular sequence register.

ADC1->CR1   = 0;                         // 12-bit resolution, independent mode

ADC1->CR2   = 0;                         // Right alignment, software trigger

ADC1->SMPR1 |= (7U << 0);               // Max sample time on CH10

ADC1->SQR3  = 10;                        // First conversion: channel 10 (PC0)

Step 4 — Enable ADC and start conversion: Once configured, the ADC is enabled and a conversion is triggered by software.

ADC1->CR2 |= ADC_CR2_ADON;              // Enable ADC1

ADC1->CR2 |= ADC_CR2_SWSTART;           // Start conversion

while (!(ADC1->SR & ADC_SR_EOC));        // Wait for end of conversion

uint16_t raw = ADC1->DR;                 // Read 12-bit result (0–4095)

To read both sensors, the sequence register (SQR3) is updated between conversions to select CH10 (PC0, Section A) and CH11 (PC1, Section B) alternately. The raw 12-bit value is then mapped to a moisture percentage: a high ADC value corresponds to dry soil, and a low value corresponds to wet soil, since the analog output voltage decreases with moisture.

1.3 Important Considerations

- The probe is susceptible to corrosion over time due to electrolysis when continuously powered. To extend its lifespan, the sensor should be powered only during measurement intervals using a GPIO-controlled transistor switch, rather than keeping VCC permanently connected.

- Operating voltage is 3.3 V to 5 V, making the sensor directly compatible with the STM32 Nucleo-64's 3.3 V logic without any level shifting.

- Calibration for the specific soil type used is recommended before deployment, as different soil compositions affect sensor resistance and therefore output voltage mapping.

- A long sample time (ADC_SMPR set to maximum, 480 cycles) is recommended for resistive sensors to allow the voltage to settle before sampling.

1.4 Key Specifications

| Parameter | Value |
| --- | --- |
| Operating voltage | 3.3 – 5 V |
| Output modes | Analog (AO) + Digital (DO) |
| Comparator | LM393 |
| Sensitivity | Adjustable via on-board potentiometer |
| ADC resolution (STM32) | 12-bit (0 – 4095) |
| Pin configuration | VCC, GND, AO, DO |

- Temperature and Humidity Sensor DHT11

  - Operating Principle

The DHT11 is a digital temperature and humidity sensor that uses a capacitive humidity element and a thermistor to measure ambient conditions, outputting both values as a serial bitstream through a single data pin. In the irrigation system, the DHT11 readings are used to adapt watering decisions based on real environmental conditions — for example, suppressing an irrigation cycle when ambient humidity is already high.

The sensor uses a proprietary single-wire digital protocol. Communication starts with a START signal from the MCU, followed by an 80 µs response from the sensor and then 40 bits of data (8-bit humidity integer, 8-bit humidity decimal, 8-bit temperature integer, 8-bit temperature decimal, 8-bit checksum). Each bit is encoded by the duration of a HIGH pulse: approximately 26–28 µs for a '0' and 70 µs for a '1'.

A pull-up resistor of 4.7 kΩ to 10 kΩ must be connected between the DATA line and the 3.3 V rail to hold the bus HIGH when idle.

  - STM32 Peripheral: GPIO

Because the DHT11 protocol is not natively supported by any STM32 hardware peripheral, it is implemented using bit-banging on a standard GPIO pin configured to switch dynamically between output and input mode during the transaction. Sensor A uses PC2 and Sensor B uses PC3, both accessible on the morpho connector CN7.

Precise microsecond timing is required. In Bare Metal, this is implemented using a hardware timer (TIM2) configured as a microsecond delay counter, rather than a software loop which would be unreliable across compiler optimization levels.

Step 1 — Enable clocks and configure GPIO: Enable GPIOC clock and initially configure the pin as output push-pull.

RCC->AHB1ENR |= RCC_AHB1ENR_GPIOCEN;

GPIOC->MODER |= (1U << 4);  // PC2 → Output (MODER bits [5:4] = 01)

GPIOC->OTYPER &= ~(1U << 2); // Push-pull output

Step 2 — Send START signal: Pull DATA LOW for at least 18 ms, then release HIGH and wait 20–40 µs for the sensor response.

GPIOC->ODR &= ~(1U << 2);   // Pull DATA LOW

delay_ms(20);               // Hold LOW for ≥ 18 ms

GPIOC->ODR |= (1U << 2);    // Release HIGH

Step 3 — Switch to input and read response: Reconfigure the pin as a floating input to receive the sensor's 80 µs LOW / 80 µs HIGH response, then read 40 data bits.

GPIOC->MODER &= ~(3U << 4); // PC2 → Input mode (MODER bits [5:4] = 00)

// Wait for sensor LOW (80 µs), then HIGH (80 µs) response pulses

Step 4 — Decode bits: For each of the 40 bits, wait for the LOW preamble (~50 µs) to end, then measure the duration of the HIGH pulse. A HIGH lasting ~26–28 µs is bit '0'; one lasting ~70 µs is bit '1'.

// Sample the pin ~40 µs after the rising edge to distinguish 0 from 1 if (GPIOC->IDR & (1U << 2)) { bit = 1; } else { bit = 0; }

Step 5 — Verify checksum: The last byte received must equal the sum of the first four bytes (masked to 8 bits). If the checksum fails, the reading is discarded.

if (data[4] == ((data[0]+data[1]+data[2]+data[3]) & 0xFF)) { /* valid */ }

Since each DHT11 reading takes approximately 250 ms to complete and a minimum interval of 2 seconds must be respected between consecutive reads, the sensor is never polled inside a blocking loop. Instead, TIM2's counter is checked at the beginning of the main loop, and a new reading is initiated only when the elapsed time exceeds 2000 ms — allowing the rest of the system to continue executing between reads.

  - Important Considerations

- Never implement the 2-second reading interval as a blocking delay (e.g., a software loop). This would stall the entire main loop and prevent the system from reading other sensors or responding to button inputs.

- The DATA pin must idle HIGH. Ensure the 4.7 kΩ – 10 kΩ pull-up resistor is connected to the 3.3 V rail, not to 5 V.

- Both the VCC supply and the pull-up resistor must use the 3.3 V rail, since the STM32 Nucleo-64 operates at 3.3 V logic.

- Bit-bang timing is sensitive to interrupts. Disable interrupts during the 40-bit reception window and re-enable them after reading is complete to avoid timing errors.

- If multiple DHT11 sensors are read, they must be read sequentially, not simultaneously, as each pin is driven independently using the same bit-bang routine called with different GPIO pin parameters.

  - Key Specifications

| Parameter | Value |
| --- | --- |
| Temperature range | 0 – 50 °C ¦ Accuracy: ±2.0 °C |
| Humidity range | 20 – 90% RH ¦ Accuracy: ±4% RH |
| Minimum reading interval | 1 – 2 seconds |
| Output protocol | Single-wire digital (proprietary) |
| Pull-up resistor | 4.7 kΩ – 10 kΩ to VCC (3.3 V) |
| Pin configuration | VCC (3.3 V), DATA, GND |

- Water Level Sensor JSN-SR04T

  - Operating Principle

The JSN-SR04T is a waterproof ultrasonic distance measurement sensor designed for outdoor and wet environments. It consists of two separate physical components: a waterproof transducer (the sensing element) connected via a 2.5 m cable to a control board. This separation allows the transducer to be submerged or mounted inside the water tank while the electronics remain in a dry enclosure. The sensor is mounted at the top of the supply tank, pointing downward, to measure the air gap between the sensor and the water surface.

Its working principle is based on time-of-flight measurement. The TRIG pin must be driven HIGH for exactly 10 µs, which commands the sensor to emit a 40 kHz ultrasonic burst. The burst travels through the air, reflects off the water surface, and returns to the transducer. The sensor then drives the ECHO pin HIGH for a duration proportional to the round-trip travel time of the sound wave. Distance is calculated as:

Distance (cm) = ECHO pulse duration (µs) × 0.01715

The factor 0.01715 comes from half the speed of sound (330 m/s ÷ 2) converted to cm/µs. The water level inside the tank is then derived as:

Water level = Tank height (cm) − Measured distance (cm)

  - STM32 Peripherals: GPIO Output (TRIG) + Input Capture (ECHO)

The TRIG pin connects to PA9 configured as a GPIO output push-pull. The ECHO pin connects to PA10 configured as Input Capture on TIM1_CH3, which allows the STM32 hardware timer to measure the ECHO pulse duration with microsecond precision without blocking the CPU.

The ECHO pin of the JSN-SR04T outputs a 5V signal. A voltage divider (1 kΩ + 2 kΩ) must be placed between the ECHO output and PA10 to bring the signal down to approximately 3.3 V, protecting the STM32 GPIO input.

Step 1 — Enable clocks: Enable GPIOA clock, TIM1 clock, and configure the alternate function mapping.

RCC->AHB1ENR  |= RCC_AHB1ENR_GPIOAEN;  // GPIOA clock

RCC->APB2ENR  |= RCC_APB2ENR_TIM1EN;   // TIM1 clock

Step 2 — Configure PA9 as GPIO Output (TRIG):

GPIOA->MODER  |=  (1U << 18);  // PA9 → Output (bits [19:18] = 01)

GPIOA->OTYPER &= ~(1U << 9);   // Push-pull

Step 3 — Configure PA10 as Alternate Function for TIM1_CH3 Input Capture:

GPIOA->MODER  |=  (2U << 20);  // PA10 → Alternate function (bits [21:20] = 10)

GPIOA->AFR[1] |=  (1U << 8);   // AF1 = TIM1 on PA10 (AFRH bits [11:8])

GPIOA->PUPDR  &= ~(3U << 20);  // No pull resistor on PA10

Step 4 — Configure TIM1 for Input Capture on CH3: Set the timer prescaler for 1 µs tick resolution, configure CH3 as input mapped to TI3, and enable both rising-edge and falling-edge captures.

TIM1->PSC  = 99;          // 100 MHz / (99+1) = 1 MHz → 1 tick = 1 µs

TIM1->ARR  = 0xFFFF;      // Max auto-reload

TIM1->CCMR2 |= (1U << 8); // CC3S = 01: CH3 mapped to TI3 (input)

TIM1->CCER  |= TIM_CCER_CC3E;   // Enable capture on CH3

TIM1->CR1   |= TIM_CR1_CEN;     // Start timer

Step 5 — Send TRIG pulse and measure ECHO: Drive TRIG HIGH for 10 µs using a TIM2-based microsecond delay, then capture the ECHO pulse duration.

GPIOA->ODR |=  (1U << 9);   // TRIG HIGH

delay_us(10);               // Hold for 10 µs

GPIOA->ODR &= ~(1U << 9);   // TRIG LOW

// Wait for rising edge on ECHO (CCR3 captures timestamp)

while (!(TIM1->SR & TIM_SR_CC3IF)); // Rising edge captured

uint32_t t_start = TIM1->CCR3;

// Switch capture to falling edge

TIM1->CCER |= TIM_CCER_CC3P;        // Capture on falling edge

while (!(TIM1->SR & TIM_SR_CC3IF)); // Falling edge captured

uint32_t t_end   = TIM1->CCR3;

uint32_t duration = t_end - t_start; // ECHO pulse in µs

float distance_cm = duration * 0.01715f;

  - Important Considerations

- The JSN-SR04T ECHO pin outputs a 5V signal. Always use a voltage divider (1 kΩ series + 2 kΩ to GND) before connecting to PA10, since STM32 GPIO pins are not 5V tolerant.

- Minimum measurable distance is 25 cm. If the tank is full and the water surface is closer than 25 cm to the sensor, readings will be invalid. The sensor mounting height must account for this.

- Add a timeout check in software: if no ECHO rising edge is detected within 30 ms after the TRIG pulse, abort and return an error code to avoid the main loop hanging indefinitely.

- The measurement cycle should be triggered periodically (every 500 ms to 1 s is sufficient for water level monitoring), not continuously, to avoid ECHO interference between measurements.

- The speed of sound varies with temperature (approximately 0.6 m/s per °C). For higher accuracy, the distance formula can be corrected using the DHT11 temperature reading: Speed (m/s) = 331.4 + (0.6 × Temperature °C).

  - Key Specifications

| Parameter | Value |
| --- | --- |
| Operating voltage | 3.0 – 5.5 V DC |
| Working current | < 8 mA ¦ Static: 5 mA |
| Probe frequency | 40 kHz |
| Measurement range | 25 cm – 450 cm |
| Distance accuracy | ±1 cm |
| Resolution | 1 mm |
| Measuring angle | 75° |
| Operating temperature | −10 °C to +70 °C |
| Cable length | 2.5 m |
| Pin configuration | VCC (5 V), TRIG (input), ECHO (output), GND |

References

- STM32F411xC/xE Reference Manual (RM0383) — STMicroelectronics. https://www.st.com/resource/en/reference_manual/rm0383-stm32f411xce-advanced-armbased-32bit-mcus-stmicroelectronics.pdf

- JSN-SR04T Waterproof Ultrasonic Sensor — ElProCus. https://www.elprocus.com/jsnsr04t-waterproof-ultrasonic-sensor/

- DHT11 Sensor Tutorial — Naylamp Mechatronics. https://naylampmechatronics.com/blog/40_tutorial-sensor-de-temperatura-y-humedad-dht11-y-dht22.html

- Soil Moisture Sensor YL-69 / HL-69 with Arduino — Random Nerd Tutorials. https://randomnerdtutorials.com/guide-for-soil-moisture-sensor-yl-69-or-hl-69-with-the-arduino/